INSPECTION REPORT

Inspector: Roger Anderson						Date: March 31 2009

Location: Dade City Fl.

Unit #  7  52 meter Schwing

START HERE

TRUCK

l.   Engine

Condition makes it not possible for visual 					inspection

Make   Detroit

Model	 	Series 60

S/N

H.P. 	 400

	Emission Label

x		Not Found

Yes

Damaged

 	Starter

x		Functional

Replaced

Noisy

Dragging

   Exhaust System

x		Functional

x		Broken or Cracked small

Loud muffler	not working.

x		Rust Penetration

Replacement

Weld repair

Broken? Loose Brackets

	Oil Leaks

None Seen

High Oil Level

x		Low Oil Level

Residue	Seepage	Leak                                 	Value Cover

  	Side of Engine

	Front of Engine	                x

	Oil Pan

 	Rear of Engine

  	Fuel Systems

x	Leaks not seen

Leaks		Seepage

Fuel Line

Injector Lines

Fuel Filter

Fuel Tanks

Dented

 	Radiator & Cooling System

x	Leaks not seen   low level

Leaks

Seepage

Radiator Hoses

Heater Hoses

Concrete spatter all over front

Radiator

x	Minor Plugging in Fins

Major Plugging of Fins

Fin Damage

Bent or Gouged Top or Bottom Tank

Fan Clutch Operational

Loose Clamps

Hard or Cracking Hoses

	Turbo or Blower

N/A

x	Functional

Oil Seepage

Oil Leaks

Exhaust Connection Leak

Rust

Appears to be a Replacement

Hoses or Clamps Loose

Makes Noise (bearing wine)

1.  Engine con’t

 	Engine Head Gasket

x	OK

Coolant Leaks

Oil Seepage

Oil Leaks

Reside from Unknown source

	Belts

x	OK

Worn

Cut

Dry Cracking

Frayed

Loose

Idler Bearing Noise

	Injector Pump

x	OK

Leaks

Seepage

Wet

Throttle Shaft Ware Sloppy

Appears to be a replacement

	Functional Engine Check

x	Engine started and ran during test

Engine started and ran with smoke that 					Disappeared

Engine ran rough with change in RPM

Engine ran with abnormal noise

Remote Engine Control was Functional

	Check Blow-By

None Seen  	 Minor  	Major x

Comments:  Needed jump to start.  Extreme amount of oil on the underneath side of unit

Engine Brake		Yes x		No

2 position

2.  	Drivelines

Loose U Joints

Greased

Vibration

x		Dry

3. 	Transmission

Make: Eaton fuller

Model: 8 speed/ compound low

Linkage Loose

Input and Output Shafts

a.   Leaks

b.   Seepage

x	Damage to Shift Tower or Knob missing top

Comment:

4.   Clutch

x	OK

x	Excessive Pedal Travel

5. 	Suspension

Front

Springs

x	Air

Comments: Flat lh  Coated with grout dirt and oil

RH shock absorber bracket bent down and forward

Rear

Spring

Solid

Walking Beam

x	Air

Cracked

Misaligned

x	Worn Bushing RH #2

Worn Rubber Biscuits

Cut Air Bag

x	All Equalize Front and Rear

Properly adjusted cables for Air Bags Front and 					Rear:   #3 axle cable broke

x	Tie Rods and Torsion Arms Bushings Loose

Welded on or Repaired

Appears to be Replaced

6.  	Wheels

Axles

Steel	Aluminum	Cast Spoke

1.			x

2.			x

3.			x

4.			x

5.			x

6.			x

7.

Comments: Dull may have been cleaned with degreaser w/ acid

7.  	Tires

Axles…Right

Make	   	Size	    	Tread      Depth mm

1.	Hercules	385/r65	Hwy		16

2.	B F Goodrich	“		“		In 11    Out

3.	Firestone	385/r65	“		In 12 	 Out

4.	Kelly		285 75r		“	In 0 	 Out

5.	Hercules	12r22.5			In  12   Out 12

6.	Sumotoma	12r 22.5			In  14    Out 14

7.

Comments: #4 tire bald

7.  Tires con’t.

Axles…Left

Make	   	Size	    Tread      		Depth mm

1.  	Goodrich	3855r65	      Hwy	9

2.	Hercules	“		“		In 18    Out

3.	Firestone	385r65	“			In  13	 Out

4.	Dayton	285/75	“		“	In 10   	 Out

5.	Bridgestone	12r22.5			In 8	 Out 7

6	Kelly		“				In 12	 Out 15

7.

Comments: #4 Scuffed bad

8.  	Hubs and Seals

x	OK

Leaks

Seepage            Broken or Cracked Site Glass

9.  	Brakes

x  	Breaks functioned during limited test

Parking Brake

x	Holds against Power

Cams Adjusted

Park valve leaking and big air leak at compressor

10. 	Steering Box

OK

Oil Leaks  	Yes 		No x

Seepage

Levers Tight

Smooth Operation

Needed fluid to make it steer

11.	Alarms

x	Did not Function

Horn Functional From Both Locations

Back up Alarm Functional

N/A

12.	External Lights

not working	Hopper Light		 	Yes 	x	No

LH Turn Signals		Yes 		No x

RH Turn Signals		Yes		No x rh r

Clearance Lights RH top, LH behind door,LH rear, RH cab

Headlights			Yes	x	No

High Beams			Yes	x	No

Brake Lights			Yes	x	No

Back Up  Lights		Yes	x	No			        		Night Light Kit		Yes		No x

Outrigger Lights		Yes	x	No

Interior Lights		Yes	x	No

13. 	Mud Flaps

x	Functional

Some Missing: Rh rear #1

Comments: RR cured and hard

14.	Cab

Bumper, Sheet Metal, Fiberglass

x	Concrete Spatter

x	Scathed

x	Dented

Doors Close Tight and Lock

x	Cracks around Door Handle

x	Rust, Corrosion  Drivers door panel lower lsft

Repaint

x	Vent door funtional

x	Faded Paint

Weld Repair

Roof Dented

Weather stripping loose:

Comments:

	Glass

x	All Good

Pitted Windshield

Functional sliders Yes x	No.  Roll-ups Yes	No				Air 	Yes	No

x	Mirrors Okay

x	Heated  wires cut both sides

x	Convex  one top and bottom of bracket

Broken or Cracks:

	Interior

Seats Functional

OK

x	Torn cover

Air Bag Leaks, Value O.K.

Broken Brackets

Interior Lights

x	OK

Some Missing

Lens Missing:

Gauges

x	Functional

x	Easy to Read

Comments:

Switches and Knobs

x	Functional

Some Missing: A/C switch

Heater/Air Conditioner

x	Functional

Blows hot and cold

Nozzles in Good Condition:  							Needs to recharged

	Compressor

Radio

N/A

x	Functional

After Market

x	Cassette

x	Missing speakers   RH & LH front

Windshield Wipers

x	Functional

x	Needs Blades

Steps

Functional

Bent Broken:

15. 	Battery and Batter Box

x	Cover Secure

Batteries Clean no Corrosion

Cables frayed/Corroded

Loose Connections

Need new batteries

16. 	Air System

na	Compressor Size

Air Dryer 			Yes 	x	No

Recovery Slow

x	Tanks Drains Functional

Air Leaks… Explain:

17. 	Concrete Pump

Water System

Tank Size      Gal. 220

Steel	x		Plastic

Location: RH rear outrigger

Cap 				Yes		No x

Rust 				Yes		No x

Dented/Welded Repair	Yes		No x

Pump Functional		Yes x		No

Spare Tanks 			Yes		No x

Hoses and Nozzles 		Yes	x	No

Hopper

Type …. Describe:

Back Splash	OK

x	Dented, Rust, Holes  Repaint

x	Covered with Concrete

Clean in Good Condition

Grate

OK

x	Excessive Wear

x	Hinged

x	Heavy Concrete Build up

Agitator

x	Functional

x	Worn rebuilt Blades

Shafts Sealed/Hold Grease

Shafts Make Noise

x	Dry

Vibrator

Functional

Location: behind hopper

Electric    x	Air

Rubber Guard

x	OK

Needs Replaced

Grease Distribution System Rear

Type

Boom

Outriggers

Concrete Value and agitator:

Lines connected

x	Dry

x	Manual

Comments:

Rear Steps and Hand Rails

x	Bent 											Weld Repairs

Broken										Shop Built  No grip tread

Loose											Missing

Clean Free of Concrete	Yes  	x	No

Decks

OK

Dented, Rust, Scratched

Grip Strips in Place

x	Clean

x	Need Attention

Cabinets

no	Doors Close and Lock

no	Clean

x	Dented, Rusted

Control System Door Functional

Control Compartment Water Tight Yes	No x

Comments:

Concrete Valve

Style		Std

S Tube

Inlet Size mm.

Outlet Size mm.

Rock Valve

x	2 Bolt M Rock

E Rock

V Rock

Long Rock

4 Bolt M Rock

Gate

Spade

Flapper

Rubber Tube

Other

Concrete Valve Ware Parts

Steel

x	Carbide Steel

Reducing Nozzles

Percent Remaining

Cutting Ring 		0	%.

Spectacle Plate 	0	%.

Mirror Plate	groved

Comments: 	Worn out Need to be replaced

Concrete Valve Shaft

OK

Grout Leakage Front

x	Grout Leakage Rear  minor

Scared/ Nicked

Mirror Plate scared/Grooved

Concrete Valve Shift Cylinders

x	Single

Double

Pins and Bushing

x	Loose/Worn

Cylinder Rod Scratched/Nicked

Accumulator

Yes	x		No

Pressure

Material Cylinders    9”

Steel 	 x  	  Chrome

Length  79”

Diameter  9”

Original Paint 		Yes     x   	 Primer

Piston Cups

na	Holds Water

Water Box

Lids Secure 			Yes	x	No

Screens Over Opening  	Yes		No x

Water refill Valve		Yes		No x

Drain Valve functional 	Yes	x	No

Oil in Water Box   		Yes	x	No					Grout Build Up		Yes	x	No

Comment:  Excessive grout build up.

Differential Cylinders

Differential Rod Seals Leak Yes	x	No

Nicks or Scratches in Differential Rods  						Original Paint         x		Primer

PTO Power Divider

x	Siebel

Fabco

Noisy

Air Valve Actuator Functional

Oil Level

Hydraulic Pumps

Main Pump

Make   Rexroth AV 107

Boom Pump

Make 	 Rexroth A11V040

Accessory Pump

Make 	Rexroth A2F028

Service Records 	no

Shop Filtration of Hydraulic Oil   no

Last Oil and Filter Change 		na

Hydraulic Motors

Leaks/Seepage

Jerky    Water pump

Appears Weak

18. 	Outriggers

Functional

Configuration

X Front Extend Rear

x	Swing Out Front/Swing Out Rear

Swing Out Front Extend Rear

S-Extend Front Swing Our Rear

Extend Front Swing Out Rear

Extend Front fixed in Rear

Manual Swing Front Manual Extend Rear

Other Describe:

Structure

X	OK

Dented

Excessive Scratches

Rolled up Foot Plate

Cracks

Welded Repair

Hydraulic Cylinders

Scratched/Nicked

Leaks  Describe

Pins and Bushing

Greased

x	Dry

Safety and Locking Pins

x	Functional

x	Worn, Bent, damaged

Outrigger Pads

none		One for every Outrigger

Good condition

Comments:  Holders bent

19. 	Boom

Current Boom Inspection		Yes		No  	x  		 	Last Inspection 	na

Configuration

Roll and Fold:  Overhead roll and fold

Z fold

Telescoping Z

Rotation

x	Rotec	370         Rack & Pinion

Slewing Motor

x	Brake Holds;

Rotec Gear     Comments:

20. 	Rear Control

Rear Panel

x	OK

Switches Missing

Non Functioning Control

Local Control Functional

Gauges Work

x	Compartment    none

Water Tight 		Yes		No x

x		Radio Functional

21. 	Remote Control

Radio Model:    Schwing Spread Spectrum

Functional

Broken Joysticks

Spare Battery

Joysticks Return to Center

Charger in Cab

Damage to Box

Condition:

Manual Boom Controls

NA

x	Functional

x	Valves Return to Neutral

Manual Valves Shadow Remote Control

x	Bent handles

21.   E-Stop’s

na	Functional

N/A

Comment: Need to replace all

Main Pressure Gage

x	Functional

22. 	Boom Arms

#1, A,   Main Arm

Visual Cracks		Rust

Weld Repairs			Dents

Scratches

Pins/Bushing Visual Movement:

#2, B,   Arm

Visual Cracks		Rust

Weld Repairs			Dents

Scratches

Pins/Bushing Visual Movement:

#3, C,   Arm

Visual Cracks		Rust

Weld Repairs			Dents

Scratches

Pins/Bushing Visual Movement

#4, D,   Arm

Visual Cracks		Rust

Weld Repairs			Dents

Scratches

Pins/Bushing Visual Movement:

#5, E,    Arm

Visual Cracks		Rust

Weld Repairs			Dents

Scratches

Pins/Bushing Visual Movement:

22.  Pipe Arm Brackets

x	Visual Cracks  #3 & #2

None Seen

Boom

Functions Smoothly

x	Fold without misalignment

Boom Cylinders

x	Functional			Leaks

Rods scratched		Seepage

x	Pins Bushing Dry

Boom Brackets and Rest

Have Rubber Pads no

x	Boom Lines up at Rest

Comments:

Boom Safety Strap

Functional			Frayed

x	Missing			Broke

23. 	Pipeline

Pipeline Condition

x	Mismatched

x	Different Paint

x	Dented

23.  Rear End Kit

Swing Elbow

Type: std

Fits Tight Aligned: yes

90 Degree Elbow

Reducer: 63”

24. 	Accessories

Manuals

x	None Seen

Service

Parts

Truck

Tool Box w/Tools

x	None Seen

Grease Gun		Clean Out

Hammer

End Hose

x	None Seen

5” End Hose

H.D. to Metric Adaptor

5”>4” Reducer

Safety Strap

4” End Hose

Spare Remote

Spare 100’ Cord		Yes	x	No

Spare Radio Remote		Yes x		No

Spare Black & White	Yes

25. 	Summary

During limited test machine was functional.

Hydraulic System was operational during limited

Check without load.

Hydraulic System preformed with wear and leaks

As noted.

Boom functioned during limited check without load.

Concrete pump cycled during limited check without

Load.

Comments: Engine Oil see attached report.

Bent fenders RH front Lh rear

Bent bumper

Weather stripping loose

Replace all E-Stops

Holding valve on boom rusted out.  Too much heat build up could indicate internal problem  replace

Pump kit is labeled as a BPL 1200 but it has twin circuit hyd controls, needs to be re labeled 1200TC

Missing driveline to front axle takenoff because they wanted to use 385 front tires due to cost.  They would not match the rear tires.  Need to be replaced.

(Engine Oil, Hydraulic Oil, Transmission, and Gear Oil analysis available upon request.)

Inspector 							Date

Owner(Representative) 					Date