Value-Centers LLC.

Phone: 334-265-6012  Fax: 334-265-6024

186 West Blvd., Montgomery, Alabama 36108

www.value-centers.com   Contact: Robert Preston   email: rpreston@value-centers.com

INSPECTION REPORT

A-2568

| CUSTOMER: | Wells Fargo |
| --- | --- |
| DATE: | 04/06/10 |

| Year: | 2007 | Engine Make: | Volvo 12.1L |
| --- | --- | --- | --- |
| Make: | Volvo | Engine Model: | VE D12-395 |
| Model: | VNM64T | HP: | 395 |
| VIN: | 4V4MC9GG37N460934 | Engine Brake: | Yes |
| Miles/Hours: | 166,553 | Transmission: | Eaton Fuller |
| ECM Miles: | 166,553 | Speeds: | 10 |
| Cab Color: | White | Axle: | Tandem |
| Interior Color: | Gray | Front Axle Cap: | 12,000 |
| Trim Level: | Standard | Rear Axle Cap: | 38,000 |
| Cab: | Air | Wheel Base: | 158in |
| Body Style: | Conventional | Tire Size: | 11R22.5 |
| Suspension: | Air Ride | Front Wheels: | Steel |
| Frame: | Single | Rear Wheels: | Steel |
| 5th Wheel: | Air Slide | Rear End Ratio: | 3.90 |
| A/C: | Yes | Power Steering: | Yes |
| Air Shield: | No | Power Windows: | Yes |
| Dual Exhaust: | No | Power Locks: | No |
| Air Cleaners: | No | Dual Bunks: | No |
| High Back Seats: | Yes | Dual Gas Tanks: | No |
| Visor: | Yes | Sunroof: | No |
| Cruise Control: | Yes | AM/FM/Cassette: | Yes |
| Chrome Bumper: | No | Heated Mirrors: | No |
| Air Ride Seats: | Driver |  |  |

| Tread Depth Measurements (32nds) | Brakes Remaining (%) |
| --- | --- |

| Steer 25% | R1 25% | R2 25% | R3 |
| --- | --- | --- | --- |

|  | 17 | 13 |  |
| --- | --- | --- | --- |
| 1 | 1 | 3 | 1 |
| 1 | 1 | 7 | 1 |

Damage and Repair Estimate

In Cab / Glass

The windshield is cracked. The interior needs to be cleaned and detailed.

Outside

The hood latch is broken and the hood will not open. The driver side door is dented. The bumper is damaged. The quarter fenders are damaged. The mud flaps are missing. The truck needs to be cleaned.

Engine & Drive Train

The engine needs to be serviced and cleaned. The batteries are dead and need to be replaced. There are multiple air leaks throughout the trucks. The clutch needs to be adjusted or replaced.

Tires & Brakes

Some of the tires are heavily worn. The brakes and drums are heavily worn.