## VALUE-CENTERS, L.L.C.

## 186 West Boulevard

Montgomery, AL  36108

334-265-6012 (OFFICE)   334-265-6024 (FAX)

www.value-centers.com

INSPECTION REPORT

A-2880

| CUSTOMER: | Wells Fargo |
| --- | --- |
| DATE: | 05/03/12 |

| MACHINE DESCRIPTION: |  |  |  |
| --- | --- | --- | --- |
| Manufacturer: | John Deere | Serial Number: | DW748GX595065 |
| Model: | 748GIII | Hour Meter: | 5,422 |
| Type: | Single Arch Skidder | Cab: | Enclosed |
| Year: | 2005 | A/C: | Yes |
| Comments: The hour meter is questionable. |  |  |  |

| ATTACHMENTS: |  |  |  |
| --- | --- | --- | --- |
| Grapple: | Bunching | Blade: | 116in |
| Winch: | Deere |  |  |
| Comments: The winch cable is damaged. |  |  |  |

| GENERAL CONDITION: VERY GOOD: ( ) GOOD: ( ) FAIR: ( ) POOR: (X) |  |  |  |
| --- | --- | --- | --- |
| Sheet Metal: | Some Damage | Controls: | Some Damage |
| Paint: | Scratched / Rust | Frame: | Good |
| Glass: | Some Damage | Gauges: | Some Damage |
| Seat: | Good | Other: |  |
| Comments: Some of the engine panels are missing. The left side door is dented. The right windshield segment is damaged. The machine needs to be cleaned. |  |  |  |

| ENGINE: |  |  |  |
| --- | --- | --- | --- |
| Make: | Deere | Water in oil: | No |
| Starting ability: | Good | Leaks: | Some |
| Smoke: | Some | Radiator: | Dirty |
| Blow by: | Some | Battery Condition: | Missing |
| Oil Pressure: | Good | Charging System: | Good |
| Oil Level: | Good | Belts: | Worn |
| Belts: | Worn |  |  |
| Comments: One of the batteries is missing. There are several engine oil leaks. The engine needs to be serviced. |  |  |  |

| AXLES, PLANETARIES, DRIVELINE, BRAKES: |  |  |  |
| --- | --- | --- | --- |
| Brake Condition: | Good |  |  |

| HYDRAULIC SYSTEM: |  |  |  |
| --- | --- | --- | --- |
| Cylinders: | Some Leaking | Controls: | Some Damage |
| Rod condition: | Good | Leaks: | Some |
| Pump: | Good | Oil Level: | Low |
| Hoses: | Some Leaking | Other: |  |
| Comments: The machine has a major hydraulic oil leak under the cab. The machine is low on hydraulic oil. |  |  |  |

| LINKAGE: |  |  |  |
| --- | --- | --- | --- |
| Center pins: | Normal Wear | Cracks or Welds: | None Visible |

| TIRES: |  |  |  |
| --- | --- | --- | --- |
| Front Tire Size: | 30.5L-32 | Rear Tire Size: | 30.5L-32 |
| Tread Percentage: |  | Other: |  |
| 0; 0. 0; 0. | Comments: All of the tires are heavily worn. |  |  |